- N-Acetylmuramic Acid (100 mg) – Part No.: A3007-100 MG (One hundred MG)

- Lipopolysaccharides from Escherichia coli O111:B4 – Part No.: L2630-10 MG (Ten Mg)

- Lipoteichoic Acid from Staphylococcus aureus – 10 mg (Ten MG)
Part No.: L2515-10MG 

LPS Sers spectra

https://pmc.ncbi.nlm.nih.gov/articles/PMC8301963/

LPS stock solutions were prepared in pyrogen-free water at a 10 nmole/mL concentration with extensive vortexing and sonication prior to each dilution

The AgNRs substrates used in this study were fabricated by an oblique angle deposition (OAD) technique.”

“AgNR substrates have a SERS enhancement factor of ~10⁹ and a batch-to-batch variation of <10%.

Unique surface-enhanced Raman scattering (SERS) spectra of LPS structures can be obtained.
No mention of:

- Antibodies

- Aptamers

- Reporters (4-MBA, PAP, etc.)

SERS spectra were collected … with a 785 nm near-IR diode laser. Laser power on the sample of 100 mW.
SERS spectra were collected … for 10 to 30 s
2 µL of the testing sample was dispensed directly on the AgNR substrate
All the LPS samples here are measured at a concentration of 10 nmole/mL, which corresponds to their relevant pathophysiological concentrations in clinical infections
Measured by an Enwave ProRaman-L-785A2 Raman analyzer (Enwave Optronics, Irvine, CA, USA).
All spectra baselines were first corrected and the average spectra from the nine different measurements per sample were presented
Table 1 (Neisseria meningitidis)

| Raman shift (cm⁻¹) | Assignment |
| --- | --- |
| 550 | β(CH₂) ring |
| 680 | δ(C-O-C), fatty acid |
| 850 | ν(C-O-C), saccharide |
| 988 | Carbohydrates (KDO) |
| 1131 | Fatty acid C-C |
| 1330 | Phospholipid δ(CH) |
| 1450 | CH₃ / CH₂ deformation |

Table 2 (Enteric LPS)

| Raman shift (cm⁻¹) | Assignment |
| --- | --- |
| 543 | Glycosidic ring |
| 552 | Lipid ring |
| 715 | C-N |
| 733 | Carbohydrate |
| 981 | Lipid β(CH) |
| 1131 | Fatty acid |
| 1330 | Phospholipid |

Wu et al. demonstrated that lipopolysaccharides (LPS) derived from Gram-negative bacteria can be directly detected and differentiated using surface-enhanced Raman scattering without the use of antibodies, aptamers, or reporter molecules. The study employed silver nanorod substrates and measured aqueous LPS samples at clinically relevant concentrations (10 nmol/mL) using a 785 nm laser with a power of 100 mW and acquisition times of 10–30 s. Distinct Raman fingerprints arising from lipid A, phospholipids, and carbohydrate moieties enabled bacterial differentiation via PCA. Importantly, the detection relied on intrinsic molecular signatures of endotoxins present in liquid samples, supporting label-free and lysis-free diagnostic sensing.
---------------------------------------------------------------------------------------------------------------------------

https://www.mdpi.com/1422-0067/24/15/12099
A Surface-Enhanced Raman Spectroscopy-Based Biosensor for the Detection of Biological Macromolecules: The Case of the Lipopolysaccharide Endotoxin Molecules
Rusciano et al., Int. J. Mol. Sci. 2023
LPS detection in aqueous solution”
“free LPS molecules from E. coli … present in aqueous solution”
a high-enhancing nanostructured silver substrate”

“coral-like nanomorphology with a strong porous character”

a 30 nm Ag film … treated by inductively coupled plasma (ICP) to induce the formation of plasmonic nanostructures”

“coral-like nanomorphology … high density of hot spots”

anti-lipid A polyclonal antibody (Ab)”

“the silver substrate was coated with Ab”

“using the anti-lipid A polyclonal antibody (Ab) as a Raman reporter
20 µL of LPS solutions … incubated for two hours”

“25 µL droplet of 10 µg/mL antibody

LPS concentrations ranging from 1 ng/mL to 10 µg/mL”

“LOD of 12 ng/mL and LOQ of 41 ng/mL”
Raman probe at 532 nm”

“laser power impinging on the substrate of 100 µW”

| Parameter | Value |
| --- | --- |
| Wavelength | 532 nm |
| Laser power | 100 µW |
| Integration time | 1 s |

Instrument Used

WiTec Alpha 300 confocal microscope

Measurement Strategy
100 spectra were recorded in a 20 × 20 µm area”

“5 raster scans”

Raman Spectral Features

| Raman shift (cm⁻¹) | Assignment |
| --- | --- |
| ~640 | C-C-C out-of-plane bending |
| ~840 | C-H bending |
| 1001 | Phenylalanine |
| ~1250 | C-N stretching |
| ~1659 | C=O stretching |
| 1449 | CH₃ asymmetric bending |
| 2883 | CH₂ stretching |
| 2933 → 2922 | CH₃ stretching (blue shift upon LPS binding) |

Data Analysis Method

Principal Component Analysis (PCA)”

“frequency-shift SERS approach”

Ultrasensitive and label-free detection of prognostic and diagnostic biomarkers of sepsis on a AgNP-laden black phosphorus-based SERS platform
Kundu et al., 2022

https://pubs.rsc.org/en/content/articlehtml/2022/sd/d1sd00057h

10 μL solution of LPS with concentrations varying from 1000 nM to 1 nM were drop-casted onto the SERS substrate

LPS is a bacterial endotoxin present in the outer cell membrane of Gram-negative bacteria.
uniquely designed SERS substrate on black phosphorus (BP) flake”

“silver nanoparticles (AgNPs) are synthesized over the BP surface

Pristine BP flake was mechanically exfoliated … immersed in the AgNO₃ precursor solution … Ag⁺ reduced to Ag⁰
Ultrasensitive and label-free detection …”

“signature Raman peaks of LPS arises only on the AgNP@BP SERS substrate

10 μL solution of LPS … drop-casted onto the SERS substrate

concentrations varying from 1000 nM to 1 nM
the enhancement in the Raman signal is not prominent beyond ∼1 nM concentration, which is the limit of detection (LOD)
Nd:YAG 532 nm laser line”

“laser power was kept at ∼2 mW

characterized by a WITec alpha 300R Raman spectrometer
The average value of the SERS enhancement factor was found to be … EF ∼2.9 × 10¹⁴ for LPS
The signature Raman peaks of LPS are observed at 1131, 1366, and 1655 cm⁻¹”

“A Raman peak at 1316 cm⁻¹ confirms the presence of aliphatic molecules”

“Raman peak at ∼2900 cm⁻¹ originated from CO and CH₃ stretching modes

LPS Raman Fingerprint Table

| Raman shift (cm⁻¹) | Assignment |
| --- | --- |
| 1131 | Lipid vibrations |
| 1316 | Aliphatic molecules |
| 1366 | Lipids |
| 1579 | Graphitic carbon |
| 1655 | Lipids (C=O) |
| ~2900 | CH₃ / CO stretching (lipid + sugar) |

The electromagnetic enhancement is dominated over the chemical enhancement

spiked clinical samples … prepared in sterile human serum”

“LOD values as low as 1 pM and 100 fM