Freely Circulating Blood Plasma Proteins for SERS

1.	C-Reactive Protein (CRP)

2.	Procalcitonin (PCT)

3.	Serum Amyloid A (SAA)

4.	Presepsin

5.	sTREM-1

6.	Matrix Metalloproteinase-9 (MMP-9)

7.	High Mobility Group Box 1 (HMGB1)

8.	Interleukin-6 (IL-6)

9.	Tumor Necrosis Factor-alpha (TNF-α)

10.     Interleukin-1β (IL-1β)

Raman and SERS Spectral Characteristics of C-Reactive Protein (CRP)

Native Raman Spectroscopy of CRP

Bergholt et al., Analyst, 2009

- Laser Wavelength: 785 nm NIR

- System: Portable Raman spectrometer

- Sample: Purified CRP in phosphate buffer and native plasma

- Key Peaks:

  - ~1003 cm⁻¹ – Phenylalanine ring breathing

  - ~1250–1300 cm⁻¹ – Amide III (C–N stretch/N–H bend)

  - ~1445 cm⁻¹ – CH₂/CH₃ bending vibrations

  - ~1655 cm⁻¹ – Amide I (C=O stretch in peptide bonds)

Insights:

- Spectra match typical protein backbone vibrations

- Partial Least Squares (PLS) regression successfully predicted CRP levels from spectral data

- No label or enhancement needed, but required relatively high CRP concentrations (µg/mL to mg/mL)

Label-Free SERS Detection of CRP

A. Kim et al., RSC Advances, 2015

- Substrate: Ag nanoparticles (NPs) on phosphocholine (PC)-SAM

- Excitation: ~633 nm (standard for Ag-PC SERS)

- Peak Highlights:

  - ~1004 cm⁻¹ – Aromatic amino acid rings

  - ~1270 cm⁻¹ – Amide III

  - ~1450 cm⁻¹ – CH₂ deformation

  - ~2800–3000 cm⁻¹ – Aliphatic C–H stretching (sensitive to CRP presence)

- LOD: 0.01 ng/mL (buffer); 0.1 ng/mL (1% serum)

- Enhancement Factor: ~10⁵–10⁶

B. Kherani et al., ACS Appl. Nano Mater., 2023

- Substrate: Citrate-reduced colloidal Ag NPs

- Excitation: 532, 633, and 785 nm

- Peak Range:

  - ~1000 cm⁻¹ – Phenylalanine

  - ~1260 cm⁻¹ – Amide III

  - ~1660 cm⁻¹ – Amide I

- Sample: Ovine serum spiked with CRP

- Detection: CRP signal extracted via ML classification with 90% accuracy

Native Raman/SERS Detection of IL-6

de la O-Cuevas et al., RSC Advances, 2019
DOI: 10.1039/C9RA01396B

- Method: Drop-Coating Deposition Raman (DCDR) + Graphene-Enhanced Raman Scattering (GERS)

- Substrate: rGO-coated silicon wafer

- Laser Wavelength: ~633 or 785 nm (red laser for protein resonance)

- Sample: Lyophilized recombinant human IL-6 and IL-6 solution

- Key Raman Peaks:

  - ~708 cm⁻¹ – C–S stretching (amino acid residues)

  - ~938 cm⁻¹ – C–C skeletal vibrations

  - ~1261 cm⁻¹ – Amide III (α-helix, C–N stretch/N–H bend)

  - ~1655 cm⁻¹ – Amide I (α-helix, C=O stretching)

- Insights:

  - First direct report of IL-6’s intrinsic vibrational fingerprints

  - Graphene enhanced the Raman signal significantly

  - Signal reproducible and distinguishable from background proteins

  - Sensitivity: <1 pg/mL on rGO substrate (~1 fg absolute limit in dried droplet)

Label-Free SERS Immunoassay for IL-6

Majdinasab et al., Int. J. Biol. Macromol., 2023
DOI: 10.1016/j.ijbiomac.2023.127560

- Method: Intrinsic SERS detection using antibody–AuNP conjugate

- Substrate: Colloidal spherical gold nanoparticles (AuNPs)

- Laser Wavelength: Likely near-IR (785 nm)

- Sample: IL-6 in high-salt phosphate buffer

- Key SERS Features:

  - No direct IL-6 peaks reported

  - Shifts in antibody SERS bands due to IL-6 binding (e.g., ν(S–S) disulfide region)

- LOD: ~0.91 pg/mL (~0.04 pM)

- Insights:

  - Binding-induced spectral shift used as detection marker

  - No external Raman label required

  - Effective in physiological salt conditions

Raman vs. SERS (Surface-Enhanced Raman Spectroscopy) — here’s a clear, concise comparison:

| Feature | Raman Spectroscopy | Surface-Enhanced Raman Spectroscopy (SERS) |
| --- | --- | --- |
| Signal Strength | Weak (intrinsically low scattering cross-section) | Strongly enhanced (up to 10⁶–10¹⁴× signal boost) |
| Mechanism | Inelastic scattering of photons by molecular vibrations | Same mechanism plus EM field enhancement from metal nanostructures |
| Enhancement Source | None | Localized surface plasmons (usually from Au, Ag, or Cu nanostructures) |
| Sensitivity | μM to mM range (requires high concentration) | nM to pM or even single-molecule sensitivity |
| Sample Requirement | Pure, concentrated samples | Can detect trace analytes in complex matrices |
| Substrate Needed | No special substrate | Requires nanostructured metallic surface (colloids, chips, etc.) |
| Label-Free Detection | Yes | Yes (label-free and labeled formats both exist) |
| Cost & Complexity | Lower setup cost | More complex due to need for nanostructure fabrication |
| Common Use Cases | Chemical analysis, materials science | Biosensing, single-molecule detection, diagnostics |

How CRP Raman Peaks Are Verified

- Reference Vibrational Databases: Many protein vibrational modes (e.g., phenylalanine at ~1003 cm⁻¹, amide I at ~1655 cm⁻¹) are well-established from decades of spectroscopy studies.

- Pure CRP Standards: Spectra of purified CRP in buffer are compared to control samples without CRP. Peaks unique to the CRP sample are attributed to its structure.

- Concentration Dependence: If a peak's intensity increases with increasing CRP concentration, it supports that the peak originates from CRP.

- Spectral Matching with Known Proteins: Peaks are matched to vibrational features common in protein backbones (amide I, III, CH stretches), validating assignments.

- Computational Simulations: Some studies use density functional theory (DFT) to simulate CRP’s vibrational spectrum and compare with experimental results.

- Multivariate Analysis: Techniques like Partial Least Squares (PLS) or machine learning confirm that CRP-specific peaks correlate strongly with its concentration in mixed samples.