https://pmc.ncbi.nlm.nih.gov/articles/PMC5618359/
Characterization and Discrimination of Gram-Positive Bacteria Using Raman Spectroscopy with the Aid of Principal Component Analysis

Raman scattering and its particular effect, surface-enhanced Raman scattering (SERS), are whole-organism fingerprinting spectroscopic techniques
The a priori SERS suspensions… were prepared by mixing 500 μL a priori prepared Ag colloid with 5 μL bacteria…”

“All the spectroscopic experiments were carried out onto MgF₂ slides, by drying 5 μL of washed bacteria/SERS suspension

The high density of the alternating sugar units of N-acetylglucosamine (GlcNAc or NAG) and N-acetylmuramic acid (MurNAc or NAM)… give the stiffness and the rigidity characteristic of the bacteria
The SERS spectra were used to identify the biochemical structures of the bacterial cell wall
Two synthesis methods of the SERS-active nanomaterials were used
The AgNPs mainly interact with the cell wall components
We are detecting outer cell wall molecular fingerprints without lysis
Renishaw inVia Raman Microscope
1% of the laser power reached the sample (0.5 mW laser power and ≈70 μW effective laser power on the sample
 Raman spectra

| Parameter | Value |
| --- | --- |
| Spectral range | 570–1800 cm⁻¹ |
| Accumulations | 10 |
| Integration time | 10 s |
| Total per spectrum | 100 s |

Each Raman spectrum is the result of 10 accumulations of 10 s
“Each SERS spectrum is a result of 3 accumulations of 5 s
Bacterial concentration

| Species | Concentration |
| --- | --- |
| L. casei | 2 × 10¹⁴ CFU/mL |
| L. monocytogenes | 8 × 10¹¹ CFU/mL |

Sample volume

| Step | Volume |
| --- | --- |
| Drop-cast volume | 5 μL |
| Ag colloid | 500 μL |
| Bacteria added | 5 μL |

Raman peaks

| Raman shift (cm⁻¹) | Assignment |
| --- | --- |
| ~707 | Adenine |
| ~1260 | Amide III |
| ~1659 | Amide I |
| 1690–1770 | C=O ester stretching |

SERS peaks relevant to NAM / cell wall

| SERS peak (cm⁻¹) | Assignment |
| --- | --- |
| ~1048–1050 | Carbohydrates / sugars |
| ~1123–1145 | CN / CC (carbohydrates, lipids) |
| ~1221 | Amide III |
| ~1427–1449 | CH₂ (lipids, peptidoglycan) |
| ~1455–1459 | Oligosaccharides |

By combining the spectroscopic methods with principal component analysis (PCA), the bacterial species discrimination was successfully carried out

---------------------------------------------------------------------------------------------------------------------
The biochemical origins of the surface-enhanced Raman spectra of bacteria: metabolomics profiling by SERS

Premasiri et al., Analytical and Bioanalytical Chemistry, PMCID: PMC4911336

https://pmc.ncbi.nlm.nih.gov/articles/PMC4911336/

The dominant molecular species contributing to the 785 nm excited SERS spectra of bacteria are the metabolites of purine degradation: adenine, hypoxanthine, xanthine, guanine, uric acid and AMP.”

“These molecules result from the starvation response of the bacterial cells in pure water washes following enrichment from nutrient rich environments.”

“All of the evidence cited above unequivocally establishes that the 785 nm excited SERS spectra of bacterial cells are dominated by the reproducible SERS contributions of the free purine nucleobases…

“Whole cells were subsequently placed on an Au nanoparticle covered SERS substrate.”

“1 µL of the resulting ~10⁹ /mL bacterial suspension was pipetted directly onto the SERS substrate…”

“SERS measurements were made after approximately five minutes when nearly all the water on the SERS substrate had evaporated.”

“Bacterial cells were harvested during log phase… washed four times with deionized water.”

“No enzymatic digestion or chemical lysis was performed.” “Extrinsic labeling is not needed…”

“Label-free SERS spectra of whole vegetative bacterial cells…”

“All SERS spectra reported here were obtained using the in-situ grown, aggregated Au nanoparticle covered SiO₂ substrate…”

“Small (2–15 particles) aggregates of monodispersed ~80 nm Au nanoparticles…”

“RM-2000 Renishaw Raman microscope employing a 50× objective…”
“Excited at 785 nm.”

“Incident laser powers in the 0.2–2 mW range…”

“~10 seconds of illumination time.”

“1 µL of the resulting bacterial suspension was pipetted directly onto the SERS substrate…”
10⁹ /mL bacterial suspension…
Each of the normalized supernatant spectra is nearly identical to the corresponding bacterial cell SERS spectrum…”

“These bacterial SERS signatures are not due to structural bacterial cell wall features…”

Dominant SERS peaks at 785 nm

| Raman shift (cm⁻¹) | Assignment |
| --- | --- |
| ~725–735 | Adenine ring breathing |
| ~965 | Purine ring mode |
| ~1335–1360 | Purine vibrations |
| ~1455 | Purine deformation |