Task 1 naa read 10 paper which related raman sers detection of naa in any medium

What laser power 
instrument 
whether chip or liquid 

https://pmc.ncbi.nlm.nih.gov/articles/PMC4911336/

When they measured SERS from whole bacteria,
and when they removed the bacteria and measured only the surrounding liquid,
the SERS signals were almost the same.
Same peaks
Same shapes
Same fingerprint

The bacterial spectra were acquired with an RM-2000 Renishaw Raman microscope employing a 50× objective and excited at 785 nm. SERS spectra were obtained with incident laser powers in the 0.2 – 2 mw range in ~10 seconds of illumination time. The observed spectra typically resulted from ~10 bacterial cells within the field of view (~100 µm2). The 520 cm−1 band of a silicon wafer was used for frequency calibration. Peak frequency precision is ± 0.5 cm−1.

As seen in this figure, each of the normalized supernatant spectra is nearly identical to the corresponding bacterial cell SERS spectrum when excited at 785 nm. Consequently, these pairwise comparisons reveal that these bacterial SERS signatures are not due to structural bacterial cell wall features and must arise from small molecules sufficiently water soluble at biological concentrations, which have been secreted from the bacterial cells and collect in the exogenous regions of these organisms. The SERS signals are larger for samples containing the cells where these molecules must be more highly concentrated

bacterial SERS spectra are due to small purine-like molecules, each of the observed bacterial SERS spectra shown in Fig. 1 was fit to a linear combination of the purine SERS spectra of adenine, hypoxanthine xanthine, guanine, uric acid and AMP. The SERS spectra of 20 µM solutions of these compounds on Au nanostructured substrates excited at 785 nm and normalized to the maximum of the adenine spectrum are shown in Fig. 5. SERS spectra will be especially sensitive to the presence of adenine due to the large relative cross-section of its 739 cm−1 band (Fig. 5). The peak vibrational frequencies are shifted slightly from those observed in bulk solution Raman (non-SERS) measurements due to interactions with the Au nanoparticle surface and local solvation environment effects, as is well known for SERS.[50–52] SERS frequencies may be even slightly further perturbed relative to SERS spectra of pure aqueous solution potentially due to local pH, ionic strength or molecular complex formation in the bacterial extracellular region.

 all bacterial vibrational features result from these small purine molecules and can also be seen in the SERS spectra of the corresponding cell supernatant (Fig. 4) no components of cell walls, i.e. peptidoglycan or any of its constituents, proteins, carbohydrates, or nucleic acids are required to explain the features evident in these bacterial SERS spectra and the dominant features in 785 nm SERS spectra can be assigned to a handful of purine molecules.
Biochemical origins of the 785 nm excited SERS spectra of bacteria

The SERS spectra of washed vegetative bacterial cells are reproducible vibrational signatures and have been shown for a number of species to be largely independent of typical growth media.[53] The purines contributing to these SERS signatures are the metabolites of purine degradation.[54] The purine metabolic pathways are a highly conserved network of enzymatic reactions in all organisms and are responsible for the degradation of nucleotides and nucleic acids.[54] The end products of these degradation processes in bacteria are the set of purines that are found to dominate the SERS spectra shown here: adenine, hypoxanthine, xanthine, guanine, uric acid and AMP. The observed SERS spectra of bacteria result from these metabolites that have been secreted during sample preparation and manipulation. The concentration of these molecules is largest in regions closest to the cells they are secreted from when placed on the SERS substrates but these molecules, which range from slightly soluble to soluble in water, are found in the supernatant solution as well

785 nm SERS of bacteria primarily reflects stress-induced extracellular purine metabolites generated during water washing, rather than structural cell wall components such as NAG or NAM. Buffer composition and washing protocols directly regulate metabolic state and signal intensity. Therefore, SERS in this project is treated as a metabolic biomarker platform sensitive to bacterial stress

-------------------------------------------------------------------------------------------------------------------------------

https://pmc.ncbi.nlm.nih.gov/articles/PMC5746774/

To prevent the cells from lysing, they need to be washed with a buffer. Of the buffers examined in a previous investigation [29], borate was the only one that did not suppress the SERS effect. Borate buffer also did not exhibit an appreciable SERS spectrum.
--------------------------------------------------------------------------------------------------------------------------------
https://www.sciencedirect.com/science/article/pii/S0924203124000237#sec0030

Escherichia coli, a type of bacteria, is unique in that its cell wall contains peptidoglycan. The peptidoglycan molecule consists of repeated units of N-acetylglucosamine and N-ethylaminoglycosamine crosslinked through β-1,4 glycosidic bonds to form a strong structure. N-acetylcytidylic acid and N-acetylglucosamine are unique biomarkers for the bacterial cell wall and serve as distinguishing components from other types of diseases [25], [26], such as fungi, viruses, and parasites. They are typically found in the form of polysaccharide chains, forming the repeating unit of N-acetyl muramic acid-β-(1,4)-N-MurNAc-GlcNAc. The most stable conformation for these two molecules is the Pyranose ring conformation [27], [28]. Pyranose conformation refers to the cyclic structure of glucose or muramic acid, which consists of a six-membered ring resembling a flattened shape similar to a fused tetrahydrofuran. Therefore, research focuses on studying the geometric parameters and verifying the Raman spectral peaks associated with this stable conformation.

The spatial structures of N-acetyl muramic acid and N-acetylglucosamine molecules were first calculated using the Gaussian09 quantum chemistry software, which is commonly used to calculate the energy, molecular orbitals, and assignments of molecular vibrations in molecules. The two dimensional chemical structures of N-acetylcytidylic acid (C11H19NO8) and N-acetylglucosamine (C8H15NO6) molecules, which are shown in Fig. 2a and b, were constructed by using the ChemDraw software.
Afterwards, the above two molecular configurations, which were formed with the Chem3D software and used as initial geometry data of N-acetyl muramic acid and N-acetylglucosamine [29], were optimized by utilizing the molecular modeling software Gaussian09. In these calculation, the density functional theory method B3LYP [30] and 6–311++G(d, p) basis set [31] were used.

Based on the above spatial geometrical parameters and reported literature [8], [13], [17], [25], [35], [36], [37], [38], the vibration modes of the characteristic peaks in the Raman spectra of N-acetyl muramic acid and N-acetylglucosamine molecules were identified in detail, 
To reduce the misdiagnosis rate, the characteristic wavelengths of N-acetyl cytosolic acid and N-acetylglucosamine molecules (3010, 3016, 3019, 3022, 3023, 3029 cm−1 in Table 7) were added as supplementary variables for the SPA-RF model, as they are biomarkers of E. coli. The spectral bands with high spectral intensity in Fig. 4, namely the 3010–3029 cm−1 range, were selected for one-way ANOVA. One-way ANOVA is a statistical method used to determine whether a factor has a significant effect on the experimental results.


the spatial structure of the marker substance peptidoglycan (N-acetyl muramic acid, N-acetyl glucosamine) of Escherichia coli was constructed using density functional theory. Based on the theoretical Raman characteristic peaks of N-acetyl muramic acid and N-acetyl glucosamine, the characteristic peak assignment of peptidoglycan Raman spectra was verified. Secondly, the Raman characteristic variables of N-acetyl muramic acid and N-acetyl glucosamine obtained by one-way ANOVA, as well as the characteristic variables obtained by SPA, were used to construct a diagnostic model 

----------------------------------------------------------------------------------------------------------------------------------

https://link.springer.com/article/10.1007/s00216-015-8950-5#Sec9


 N-acetylmuramic acid were purchased from Sigma-Aldrich (Germany). Nutrient agar (NA) and hydroxylamine hydrochloride were purchased from Merck (Germany). Aqueous solutions were prepared using double-distilled, deionized water from Millipore DirectQ-UV system.

Figure 1 shows the comparison of the SERS spectra of 24 h grown E. coli, adenine, peptidoglycan of B. subtilis, N-acetylmuramic acid (NAM), and human serum albumin (HSA). At the first glance of the figure, the resemblance of the spectral patterns of NAM and HSA can be easily seen. NAM is a component of bacteria cell wall and is composed of lactic acid and N-acetylglucosamine (NAG) through an ether bond, which is also a component of bacteria cell wall. Mainly, it consists of a pyranose ring, an acetyl group bound to the ring with amide bond, and a carboxyl containing lactic acid.

Since both NAM and HSA possess amide bond, their spectral patterns between 1200 and 1700 cm−1 are similar.

 pyranose ring of NAM also has the same bands, which are also consistent with the literature 

Raman spectra of the molecules are enhanced by surface plasmons of AgNPs if they are in close vicinity of AgNPs (1–4 nm). On the other hand, thickness of bacteria cell wall is much more than this. Therefore, the origins of the bands can be the outer surface of bacteria [40]. Gram-positive (G+) bacteria have thick peptidoglycan layer along with proteins in their cell wall


Raman instrumentation
Raman microscopy system (InVia Reflex, Renishaw, UK) equipped with a diode laser at 830 nm and 50× objective. The system was automatically calibrated against a silicon wafer peak at 520 cm−1. Laser power of 0.3–3 mW was used for all measurements. The exposure time for each SERS measurement was 10 s with one accumulation.

--------------------------------------------------------------------------------------------------------------------------------------
https://pubs.acs.org/doi/10.1021/ac034689c

- the cell walls of Enterococcus spp. are formed from peptidoglycan, which is rich in N-acetyl-d-glucosamine (NAG) and N-acetylmuramic acid. NAG is also present on the cell surface of Proteus, where it is produced as a component of an acidic capsular polysaccharide. 44 Raman spectra of NAG collected in this laboratory did not exhibit a peak at this wavenumber shift (data not shown); however, Raman peaks in this region from microbes have been attributed to vibrational modes of polysaccharides. 45 The SERS spectrum of NAG taken in this laboratory (Figure 4) shows an intense peak at 730 cm-1, 

Raman Microscopy. SERS spectra were collected using a Renishaw 2000 Raman microscope (Renishaw Plc., Old Town, Wotton-under-Edge, Gloucestershire, U.K.) with a low-power (24 mW) near-infrared 785-nm diode laser with power at the sampling point ∼2 mW. 33,34 The instrument was wavelength calibrated with a silicon wafer focused under the 50× objective and collected as a static spectrum centered at 520 cm-1 for 1 s.
Liquid silver colloid (Ag nanoparticles in solution)

--------------------------------------------------------------------------------------------------------------------------------------

https://pdfs.semanticscholar.org/6a09/7d39d8f51394067a5123989b22a4165a8570.pdf

Peptidoglycan, a major constituent of the bacterial cell wall composed of sugars and amino acids, is prominently targeted in SERS due to its abundance and structural significance in maintaining cell integrity
Identifying unique SERS spectra depends on the Raman wave shift, where each peak is assigned to a corresponding structural element… mainly from the outer structure of the bacterial cell, such as the outer membrane and cell wall.
Most studies provided evidence that bacteria’s SERS signals originate from their outer envelopes.”
“The features observed in bacterial SERS spectra depict the bacterial surface

--------------------------------------------------------------------------------------------------------------------------------------

https://onlinelibrary.wiley.com/doi/10.1155/2011/989504
 Several of the bands observed were from glycosidic ring of NAG and NAM at 753 cm−1, C–N and C–C stretching at 1130 cm−1, signals from lipid content, =C–C= at 1144 cm−1 and C=C at 1588 cm−1.

Raman Microspectrometer, model RM-2000, CCD camera, Leica LX microscope, 532 nm laser line, 10–50 s exposure time, 3 accumulations and a 10x objective lens in a spectra range of 100–3000 cm−1 (Renishaw Inc., Hoffman Estates, Ill, USA). 

200 μL of colloidal NPs suspensions

Table 1

| Vibration mode | a | b | c |
| --- | --- | --- | --- |
| glycosidic ring of NAG and NAM | 753 | 732 | 728 |