https://pmc.ncbi.nlm.nih.gov/articles/PMC12756854/
High-Wavenumber Raman Spectroscopy for the Identification and Biochemical Characterization of Microbial Species

This work investigates the effectiveness of utilizing the low-background high-wavenumber region (2800–3800 cm⁻¹) to both identify and biochemically characterize microbial species.
1 μL droplets of the pellet were deposited onto a microscope slide coated in standard aluminum foil and allowed to dry.
The growth media was washed from the planktonic bacteria… resuspending the pellet in sterile phosphate-buffered saline
Spectral unmixing was performed using basis spectra from proteins, nucleic acids, carbohydrates, lipids, and cell wall components.”

“Purified peptidoglycan was used… Additionally, N-acetylmuramic acid (MurNAc) was included independently.”

High-wavenumber Raman measurements… were acquired using a Renishaw inVia Raman microscope using a 785 nm excitation laser and a 1200 lines/mm grating.

| Parameter | Value |
| --- | --- |
| Microscope | Renishaw inVia Raman |
| Laser wavelength | 785 nm |
| Grating | 1200 lines/mm |
| Spectral range | 2760–3500 cm⁻¹ |
| Spectral resolution | ~1 cm⁻¹ |

The objective supplied approximately 80 mW of power at the sample.”

“A total integration time of 50 s (10-s exposure, 5 accumulations) was used.

1 μL droplets of the pellet were deposited…
The −CHₓ stretching band (2770–3100 cm⁻¹) was isolated for analysis
. Raman Peak Assignments

| Raman Shift (cm⁻¹) | Assignment |
| --- | --- |
| 2845–2855 | –CH₂ symmetric (lipids) |
| 2870–2900 | –CH₂ asymmetric |
| 2910–2930 | –CH (carbohydrates) |
| 2930–2940 | –CH₃ symmetric (proteins) |
| 2950–2970 | –CH₃ asymmetric |
| 3000–3010 | =CH (unsaturated lipids) |
| 3055–3065 | Aromatic =CH (proteins) |

Cell wall type (100%), genus (98.9%), and species (97.4%) accuracy.